| Specification | 10g | 20g | 30g |
| --- | --- | --- | --- |
| Model | QRR0.01G/S | QRR0.02G/S | QRR0.03G/S |
| Agent dosage | 10 g | 20 g | 30 g |
| Protected volume* | Up to 0.1 m³ | Up to 0.2 m³ | Up to 0.3 m³ |
| Aerosol generation duration | ≤6 s | ≤6 s | ≤6 s |
| Service life | 10 years | 10 years | 10 years |

| Specification | Details |
| --- | --- |
| Activation method | Thermal activation; no external power required |
| Activation temperature | 170°C |
| Activation response | Aerosol generation starts within 15 seconds after reaching the activation temperature |
| Mounting options | 35 mm DIN-rail mounting or customized 3M adhesive side mounting |
| Housing color | Red |
| Thermal probes | Two probes, located at the top and bottom |
| Probe cable length | Standard: 10 cm per probe; customizable from 10 to 50 cm |
| Operating temperature | −50°C to +90°C |
| Humidity tolerance | <95% RH, non-condensing |